NTI – IC Digital Design using FPGA

SPI master/slave in HDL verilog

Supervised by: Eng. Ahmed AlHussieny

# TEAM MEMBERS:

# Esther George Fayez Abdo

# Eyad Khaled Mabrouk Noah

# Radwa Ahmed Abd-Elsatar Ali

# Abstract

This project presents the design, implementation, and verification of a configurable Serial Peripheral Interface (SPI) communication system using Verilog HDL. The system consists of a parameterized SPI Master, an SPI Slave, and an SPI RAM module integrated through a wrapper. The SPI Master supports configurable data width, SPI mode selection, clock division, and selectable transmission order (MSB-first or LSB-first). The SPI Slave receives serial commands from the master, decodes the requested operation, and communicates with the RAM to perform memory read and write transactions. Functional verification was carried out through comprehensive simulation using self-checking testbenches.

# 1. Introduction

The Serial Peripheral Interface (SPI) is one of the most widely used synchronous serial communication protocols in embedded systems and digital integrated circuits. It provides high-speed full-duplex communication between a master device and one or more slave devices using a small number of signal lines.

Unlike asynchronous protocols such as UART, SPI uses a dedicated clock generated by the master, allowing higher communication speeds and deterministic timing.

This project implements an SPI communication system completely in Verilog HDL suitable for FPGA implementation.

# 2. Project Objectives

The objectives of this project are:

- Design a configurable SPI Master.

- Design an SPI Slave capable of decoding SPI commands.

- Interface the slave with an internal RAM.

- Support configurable SPI parameters.

- Verify correct data transmission using simulation.

- Produce synthesizable RTL suitable for FPGA implementation.

# 3. SPI Protocol Overview

SPI communication uses four primary signals:

| Signal | Direction | Description |
| --- | --- | --- |
| SCLK | Master → Slave | Serial clock generated by the master |
| MOSI | Master → Slave | Master Out Slave In |
| MISO | Slave → Master | Master In Slave Out |
| CS/SS | Master → Slave | Active-low Slave Select |

Communication begins when the master asserts the Chip Select (CS) line low. The master then generates clock pulses while transmitting data through MOSI and simultaneously receiving data from the slave through MISO.

# 4. System Architecture

The implemented SPI system consists of three major modules:

The master initiates all communication while the slave interprets incoming commands and accesses the RAM accordingly.

# 5. SPI Master Design

## 5.1 Features

The SPI Master was designed to be fully parameterized.

Supported parameters include:

| Parameter | Description |
| --- | --- |
| DATA_WIDTH | Number of transmitted bits |
| MODE | SPI Mode (CPOL/CPHA) |
| CLK_DIV | Serial clock divider |
| MSB_FIRST | Transmission order |

This allows the module to be reused in different SPI systems without modifying the RTL code.

## 5.2 Finite State Machine

The SPI Master operates using a four-state finite state machine.

IDLE

- Waits for the Start signal.

- CS remains high.

- SCLK remains at idle polarity.

LOAD

- Loads transmit data into the shift register.

- Clears receive register.

- Initializes counters.

- Activates Chip Select.

TRANSFER

During this state:

- Clock divider generates the SPI clock.

- SCLK toggles according to CLK_DIV.

- MOSI shifts one bit every transmission edge.

- MISO is sampled every reception edge.

- Bit counter tracks transmitted bits.

Transmission continues until all bits have been transferred.

FINISH

- Stores received data.

- Returns SCLK to idle polarity.

- Deasserts CS.

- Raises Done flag.

- Returns to IDLE.

Flowchart of Master FSM

# 6. Clock Divider

The SPI clock is generated internally from the FPGA system clock.

Instead of toggling SCLK every system clock cycle, a counter counts from

0 → CLK_DIV-1

When the counter reaches

CLK_DIV-1

the counter resets and SCLK toggles.

Example:

For

CLK_DIV = 4

System Clock:

## ↑  ↑  ↑  ↑  ↑  ↑  ↑  ↑

Counter:

0 1 2 3 0 1 2 3

SCLK:

## ______----____----

Therefore,

SPI Clock Frequency

## F_SPI = F_CLK / (2 × CLK_DIV)

The divider allows communication with slower slave devices while keeping the FPGA operating at a much higher frequency.

# 7. SPI Slave Design

The SPI Slave receives serial data from the master and interprets incoming packets.

The slave consists of two major parts:

- SPI Receiver

- RAM Interface

The slave continuously monitors the Chip Select line.

Once selected, it samples incoming MOSI data using the serial clock provided by the master.

## Slave States

The implemented slave includes the following states:

| State | Function |
| --- | --- |
| IDLE | Wait for CS assertion |
| CHECK | Decode transaction type |
| WRITE | Receive write command |
| READ_ADDR | Receive read address |
| READ_DATA | Transmit requested data |

The slave reconstructs serial data into a parallel packet before forwarding it to RAM.

# 8. RAM Module

The RAM module stores user data.

Supported operations include:

| Command | Function |
| --- | --- |
| 00 | Write Address |
| 01 | Write Data |
| 10 | Read Address |
| 11 | Read Data |

The RAM stores 8-bit data across 256 memory locations.

9. Data Packet Format

The implemented SPI packet contains ten bits.

## ---------------------------------

## | CMD[1:0] | DATA[7:0] |

## ---------------------------------

Where:

00 → Write Address

01 → Write Data

10 → Read Address

11 → Read Data

This packet structure simplifies communication between the SPI slave and RAM.

# 10. Verification

Functional verification was performed using a self-checking Verilog testbench.

The following test cases were evaluated:

- Reset functionality

- Multiple transmit patterns

- All zeros

- All ones

- Alternating bits

- Random data

- Variable DATA_WIDTH

- Variable CLK_DIV

- Clock edge verification

Loopback testing connected

MOSI → MISO

allowing transmitted data to be immediately received and compared automatically.

Testbench for master

Testbench for read_data case in slave

Testbench for read_addr case in slave

## Testbench Features

The verification environment automatically:

- Generates system clock.

- Generates reset sequence.

- Starts SPI transactions.

- Counts SCLK edges.

- Checks received data.

- Reports PASS/FAIL automatically.

- Monitors internal FSM states.

Pass/fail reports in master testbench

# 11. Results

Simulation demonstrated successful SPI communication under multiple operating conditions.

The SPI Master correctly:

- Generated the SPI clock.

- Shifted transmitted data.

- Sampled incoming data.

- Controlled Chip Select.

- Produced correct Busy and Done signals.

The SPI Slave successfully:

- Decoded SPI commands.

- Received serial packets.

- Interfaced with RAM.

- Returned requested data through MISO.

No functional errors were observed during loopback verification.

# 12. Advantages of the Design

The implemented design provides several advantages:

- Fully parameterized architecture

- Synthesizable RTL

- Modular design

- Configurable SPI clock

- Configurable data width

- Supports MSB-first and LSB-first transmission

- Easily reusable in larger FPGA systems

# 13. Future Improvements

Several enhancements can be incorporated into future versions:

- Full support for all four SPI modes (CPOL/CPHA)

- Multiple slave selection

- FIFO buffering

- Interrupt generation

- DMA support

- Burst transfers

- CRC/Error detection

- Configurable frame sizes larger than 32 bits

# 14. Conclusion

A complete SPI communication system was successfully designed and verified using Verilog HDL. The project demonstrated the implementation of a configurable SPI Master, an SPI Slave, and an integrated RAM interface capable of executing serial read and write operations. Through parameterization and modular RTL design, the architecture can be adapted to different communication requirements and FPGA platforms. Comprehensive simulation verified correct clock generation, serial data transmission, packet decoding, and memory access, demonstrating the functionality and reliability of the proposed design. The project also provides a scalable foundation for implementing more advanced SPI features such as support for all SPI modes, multiple slave devices, burst transfers, and FIFO-based buffering in future developments